Pedagogy REPORT

Title of the Experiment

Design Verilog program to implement types of De-Multiplexer  4:1 and 8:1

Course

Digital Design and Computer Organization Pedagogy Activity

Objective

To design, implement, and simulate 4:1 and 8:1 Demultiplexer circuits using Verilog HDL, and to verify their functionality through simulation and waveform analysis.

Tools and Software Used

- Verilog HDL for hardware description

- Icarus Verilog for compilation and simulation

- GTKWave for waveform visualization and signal analysis

GTKWave is used to observe the timing behavior of input and output signals and to validate the correctness of the demultiplexer operation.

Prepared a detailed report on the executed program, including:

- Source code

- Explanation of the logic

- Output obtained

- Observations or conclusions

1. 4:1 Demultiplexer

Aim

To design and simulate a 4:1 Demultiplexer using Verilog HDL and verify its functionality using a testbench.

Introduction

A demultiplexer (DEMUX) is a combinational logic circuit that takes one input signal and routes it to one of several output lines based on the values of select inputs.
In a 4:1 demultiplexer, one input is routed to one of four outputs using two select lines.

Source Code:

Logic Explanation

Inputs

- I – Data input

- S0, S1 – Select lines

Outputs

- Y0, Y1, Y2, Y3

Each output corresponds to one unique combination of select lines.

Truth Table Logic

| S1 | S0 | Active Output |
| --- | --- | --- |
| 0 | 0 | Y0 = I |
| 0 | 1 | Y1 = I |
| 1 | 0 | Y2 = I |
| 1 | 1 | Y3 = I |

Only one output is active at a time, while others remain 0.

Boolean Logic Used

Each output is enabled only when its select condition is satisfied.

Y0 = (~S0) & (~S1) & I;

Y1 = (~S0) & ( S1) & I;

Y2 = ( S0) & (~S1) & I;

Y3 = ( S0) & ( S1) & I;

- ~S0 or ~S1 ensures the select line is LOW

- & I ensures output depends on the input

- Logical AND ensures exclusive activation

This implementation is purely combinational and does not require clocks or memory elements.

Testbench Explanation

The testbench:

- Generates all possible combinations of select lines

- Keeps input I = 1

- Observes which output becomes HIGH

The $monitor statement prints signal values for each time step, allowing verification of correct routing.

Output:

Observations / Conclusions (4:1 DEMUX)

- For each select combination, only one output is HIGH

- All other outputs remain LOW

- The demultiplexer correctly routes the input to the selected output

- The design is free from race conditions and glitches

- Simulation results match the theoretical truth table

2. 8:1 Demultiplexer

Aim

To design and simulate an 8:1 Demultiplexer using Verilog HDL and verify its operation through simulation.

Introduction

An 8:1 demultiplexer routes a single input signal to one of eight outputs using three select lines.
It is an extension of the 4:1 demultiplexer and demonstrates scalable digital logic design.

Source Code:

Logic Explanation

Inputs

- I – Data input

- S0, S1, S2 – Select lines

Outputs

- Y0 to Y7

Each output corresponds to one unique 3-bit select combination.

Truth Table Logic

| S2 | S1 | S0 | Active Output |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Y0 |
| 0 | 0 | 1 | Y1 |
| 0 | 1 | 0 | Y2 |
| 0 | 1 | 1 | Y3 |
| 1 | 0 | 0 | Y4 |
| 1 | 0 | 1 | Y5 |
| 1 | 1 | 0 | Y6 |
| 1 | 1 | 1 | Y7 |

Boolean Logic Used

Each output is activated by a unique AND combination of select lines and the input.

Y0 = (~S0) & (~S1) & (~S2) & I;

Y1 = (~S0) & (~S1) & ( S2) & I;

Y2 = (~S0) & ( S1) & (~S2) & I;

Y3 = (~S0) & ( S1) & ( S2) & I;

Y4 = ( S0) & (~S1) & (~S2) & I;

Y5 = ( S0) & (~S1) & ( S2) & I;

Y6 = ( S0) & ( S1) & (~S2) & I;

Y7 = ( S0) & ( S1) & ( S2) & I;

Each output:

- Is mutually exclusive

- Depends strictly on select lines

- Acts as a decoded AND gate

Testbench Explanation

The testbench:

- Cycles through all 8 combinations of select inputs

- Keeps I = 1 to observe output behavior

- Verifies one-hot output behavior

Output:

Observations / Conclusions (8:1 DEMUX)

- Exactly one output is HIGH for each select combination

- No overlap or multiple outputs active

- Output transitions are clean and deterministic

- Logic scales correctly from 4:1 to 8:1

- Simulation confirms theoretical expectations